How to Configure DS-K1T502 as Main and Sub Door Station on Web Page

DS-K1T502DBFWX-C * 2

沙木 Sam

Applicable to

- Pro Fingerprint Terminal ：DS-K1T502DBFWX-C *2

How to Configure DS-K1T502 as Main and Sub Door Station on Its Web Page

Key Step 1: Configure one of the DS-K1T502DBFWX-C terminal as sub door station.

- Please log in to one of the DS-K1T502DBFWX-C’s web page

- Please configure this DS-K1T502DBFWX-C as a sub door station terminal (For the convenience of distinguishing, use A to represent this terminal), follow these steps:

  - Please go to Configuration→ Intercom → Device No. → Door Station No change to number 1 (Means terminal A Configured as a sub door station)

Picture 1

  - Shows Rebooting after click Save, and then automatically exit the device’s web page by itself.

Picture 2

  - Then please login to terminal A’s web page again, shows there is no Device management bar, means terminal A already configured as a sub door station.

Picture 3

- Please register the main door station DS-K1T502DBFWX-C terminal (For the convenience of distinguishing, use B to represent this terminal) to terminal A sub door station terminal, follow these steps:

  - Please go to the Configuration→ Intercom → Session Settings, fill in the main door’s Registration Password and Main door station IP (terminal A’s) → Click Save → will shows Save Succeeded

Picture 4

Key Step 2: Register sub door station terminal to the main door station terminal

- Please log in to the terminal B’s web page

- Please configure terminal B as a main door station terminal, follow these steps:

  - Please go to Configuration→ Intercom → Device No. → Door Station No → keep it to default number 0 (Means this terminal Configured as main door station terminal)

Picture 5

- Please register sub door station terminal A to the main door station terminal B, follow these steps:

  - Please go the Device Management → Device List → Click Add Icon

Picture 6

  - Please fill in the corresponding sub door station terminal B’s information to the Edit Device Pop-ups page, and please remember that the Device Type have to be selected as Sub Door Station.

Picture 7

  - Check the sub door station’s Network Status is Online or Offline, if the Network Status shows Online means registration succeeded.

Picture 8