Preparation

- A door stations or a villa door station (DS-KV6113 in this document);

- Client software iVMS-4200 (V3.6.1.5 bulid20210831 in this document).

- Card issuer DS-K1F100

## Step 1: Add Devices to iVMS-4200

Click Online Device tab to find device and then add the device  to iVMS-4200

## Step 2: Set the door station in iVMS-4200

Choose the door station, click Remote Configuration, Then go to Intercom > Access Control and Elevator Control  , click enable Card Encryption Pararmeters, and click Save.

## Step 3: Encrypt  cards in iVMS-4200

- Add person

- Click item “card”

- Click “Settings” to set the cards

Open the M1 Card Encryption, and select the Sector 1, then click OK

Put the card on the issuer, and click Read to get the card’s number, then click Add

And put the information in Resident Information

## Step 4: Issue cards to door station

Choose Access Control  >  Authorization  >  Access Group  >  Add , to add a new access group and save it ,then  apply to the device .

First Choice for Security Professionals

HIKVISION Technical Support